1-2.Light up multiple colorful lights on body of robot

1.Preparation

1.You should learn about the position of the colorful lights in the body of hellobot；

2.You should learn about the micro:bit pins connected to the colorful lights in the schematic.

1-2-1 colorful lights on body of hellobot（back）

1-2-2 colorful lights on body of hellobot

We can make different colors of light and brightness by programming.

1-2-3 schematic

1-2-4  Pins of Micro:bit

From the schematic diagram in P1-1-3, you can see that the colorful lights is connected to the P16 of the Micro:bit.

Note:In the bottom layer of the HelloBot package has been set parameter for the user, you can directly drag the colorful lights building blocks.

- Learning goals

In this lesson, we will learn to light up four colorful lamps in turn, the colorful lamps will emit light of different colors.

3.Programming

3.1 Programming online

1) You should use the USB cable to connect the micro:bit to the computer, at this point, the computer will have a micro:bit U disk. You need to open it, click micro:bit website, then entered the micro:bit website or you can enter the URL directly in your browser: http://microbit.org/

2) After entering the programming interface, you need to click Add package and copy the HelloBot package URL: https://github.com/lzty634158/HelloBot to the input field, click to confirm the add package. Then you can use the blocks of the HelloBot package.

3.2 Programming offline

1) You can double-click to use it. As shown in the following figure.

2) After entering the programming interface, you need to click Add package and copy the HelloBot package URL: https://github.com/lzty634158/HelloBot to the input field, click to confirm the add package. Then you can use the blocks of the HelloBot package.

Note: The package only needs to be added once. If you have added packages in the previous lessons, this course does not need to be added repeatedly.

1-2-5 total program

The locations of blocks in the total program are shown in the following figure.

1-2-6

1-2-7

1-2-8

4.Download programming

You need to make sure that the micro:bit development board is connected to the computer. Then you should click on the download in the lower left corner as shown in P 1-2-9 to download the program to micro:bit.

1-2-9

- Phenomenon

After the code is uploaded. You can see that four colorful lamps are lit up in turn.

1-2-10                              1-2-11

1-2-12                             1-2-13