- 1-3.Breathing light on body of robot

- 1.Preparation

- 1.You should learn about the position of the colorful lights in the body of hellobot；

- 2.You should learn about the micro:bit pins connected to the colorful lights in the schematic.

1-3-1 colorful lights on body of hellobot（back）

- 1-3-2 colorful lights on body of hellobot

- We can make different colors of light and brightness by programming.

- 1-3-3 schematic

- 1-3-4 Pins of Micro:bit

- From the schematic diagram in 1-3-3, you can see that the colorful lights is connected to the P16 of the Micro:bit.

- Note:In the bottom layer of the HelloBot package has been set parameter for the user, you can directly drag the colorful lights building blocks.

- Learning goals

- In this course, we learns the new game of colorful lamps - breathing lights. Breathing light: the colorful lamps are changed gradually from light to dark by the control of the micro:bit. It feels like a person is breathing.

- 3.Programming

- 3.1 Programming online

- You can enter the URL directly in your browser: http://python.microbit.org/v/1 to enter programming online interface.

- 3.2 Programming offline

- 1) You can double-click to use it. As shown in the following figure.

- 2) After entering the programming interface.

- 1-3-5

Note:

1 - The capital letter / lowercase letters must be distinguished！

2 - Correct spelling!

3 - Keywords such as # need a space between the content.

4 - The program ends with a blank program.

- 5 - The block body (such as the body of the while is marked by indentation), compared to the C language, Python completely eliminates the braces (along with the semicolon of the suffix), and uses the indentation structure to represent the relationship. You can only use the Tab key (tabulation key) for indentation.

- 4.Download programming

- You need to make sure that the micro:bit development board is connected to the computer and you need to click “Flash” button.

- 1-3-6

- 5.Phenomenon

- After the code is uploaded. You can see that the colorful lamps of robot are changed gradually from light to dark by the control of the micro:bit. It feels like a person is breathing. As shown in the following figure.

- (b)

- (c)                                 (d)

- 1-3-7