- 8-2.Avoid_following

- 1.Preparation

- 1.You should learn about the position of the infrared module in the body of hellobot；

- 2.You should learn about the principle of the infrared obstacle avoidance.

- 8-2-1 RGB tracking obstacle avoidance 3 in 1 module

- 8-2-2 RJ45 interface of RGB tracking obstacle avoidance 3 in 1 module

- (b) infrared obstacle avoidance probe

- 8-2-2 RGB tracking obstacle avoidance 3 in 1 module

- 8-2-3 about wiring

- The RGB tracking obstacle avoidance 3 in 1 module is installed under the robot arm of the HelloBot, and we need to use the network cable to connect the RGB tracking obstacle avoidance 3 in 1 module.

- 8-2-4 schematic

- 8-2-5 schematic of motor drive

- 9-2-6 schematic of PCA9685PW

- 9-2-7  Pins of Micro:bit

- From the schematic diagram. You can see that Pin _Trig of ultrasonic module(SCL) is connected to P3 of Micro:bit, Pin_Echo of ultrasonic module(SDA) connected to P4 of Micro:bit.

- PCA9685PW connects micro:bit P19 and P20 pins for I2C communication.

Principle of infrared obstacle avoidance:

The basic principle of the infrared sensor to avoid obstacles is to use the reflective nature of the object. Within a certain range, if there is an obstacle, the infrared rays will encounter obstacle and will be reflected to reach the sensor receiving pin.

! Note: 
The infrared obstacle avoidance sensor use pins P3 and P4 of micro:bit . They are multiplexed with the pins of the micro:bit LED dot matrix. Before using these functions, we can use microbit.display.off() to turn off the micro:bit LED dot matrix display.

Note:The infrared obstacle avoidance sensor is not working properly due to the interference of outdoor light. This course needs to be carried out indoors and the curtains are covered to block the outdoor light.

- 2.Learning goals

- You need to put the HelloBot robot in a spacious field, use the paper or hand to block the infrared sensor in front, the indicator light is off, and the robot follows the object in front. When only the object is detected on the left side, the left indicator light is off, and the robot turn left in place to follow the obstacle. When only the object is detected on the right side, the light on the right side is off, and the turn right in place to follow the obstacle. When there is no object in front, the indicator light is on and the car stop.

- 3.Programming

- 3.1 Programming online

- You can enter the URL directly in your browser: http://python.microbit.org/v/1 to enter programming online interface.

- 3.2 Programming offline

- 1) You can double-click to use it. As shown in the following figure.

- 2) After entering the programming interface.

- 8-2-8

- 4.Download programming

- You need to make sure that the micro:bit development board is connected to the computer. Then you should click on the download in the lower left corner as shown in P 8-2-9 to download the program to micro:bit.

- 8-2-9

- 5.Phenomenon

- You need to put the HelloBot robot in a spacious field, use the paper or hand to block the infrared sensor in front, the indicator light is off, and the robot follows the object in front. When only the object is detected on the left side, the left indicator light is off, and the robot turn left in place to follow the obstacle. When only the object is detected on the right side, the light on the right side is off, and the turn right in place to follow the obstacle. When there is no object in front, the indicator light is on and the car stop.

- 8-2-10