- 2.wireless networking foundation (send digital)

- 1.Preparation

- 1.You need to prepare two HelloBot cars.

- 2.You should learn about these blocks.

- Functions of wireless blocks:

- This block is to set the group ID of the wireless communication, and can only listen to a group ID at any time.

- This block is to read the received digital packets. When the packet is received wirelessly, you can use this block.

- This block is to read the received name of variable and value of name. When the packet is received wirelessly, you can use this block.

- This block is to read the received character string packets. When the packet is received wirelessly, you can use this block.

- Send the number 0 wirelessly.

- Send the name of variable wirelessly, name is 0.

- Send the character string wirelessly.

- Learning goals

- In this course, we will learn how to use the wireless networking function. We need to write a program to set the wireless communication group ID to 1. When the button A of one of HelloBot is pressed, it will send the number 0, if another car receive digital signal, it will display 0 on the dot matrix.

- 3.Programming

- 3.1 Programming online

- 1) You should use the USB cable to connect the micro:bit to the computer, at this point, the computer will have a micro:bit U disk. You need to open it, click micro:bit website, then entered the micro:bit website or you can enter the URL directly in your browser: http://microbit.org/

- 2) After entering the programming interface, you need to click Add package and copy the HelloBot package URL: https://github.com/lzty634158/HelloBot to the input field, click to confirm the add package. Then you can use the blocks of the HelloBot package.

- 3.2 Programming offline

- 1) You can double-click to use it. As shown in the following figure.

- 2) After entering the programming interface, you need to click Add package and copy the HelloBot package URL: https://github.com/lzty634158/HelloBot to the input field, click to confirm the add package. Then you can use the blocks of the HelloBot package.

- Note: The package only needs to be added once. If you have added packages in the previous lessons, this course does not need to be added repeatedly.

- 2-1 total program

- The locations of blocks in the total program are shown in the following figure.

- 2-2

- 2-3

- 2-4

- 4.Download programming

- You need to make sure that the micro:bit development board is connected to the computer. Then you should click on the download in the lower left corner as shown in P 2-5 to download the program to micro:bit.

- 2-5

- 5.Phenomenon

- You need to download code to two Micro:bit boards.

- The experimental phenomenon of the program is that after starting up, two Micro:bit boards start networking, and their group ID is 1. When one of the Micro:bit board buttons A is pressed, which represents the sending of data signals. When another micro:bit board receives the wireless digital signal. The screen will display number 0.

- 2-6 Press the A button of one of micro:bit, another micro:bit under the same group ID will display number “0” on the dot matrix

- 2-7 Press the A button of one of micro:bit, another micro:bit under the same group ID will display number “0” on the dot matrix